# NI Circular Minerals Policy Model — Technical Documentation

Version: June 2026 · Repository: ni-circular-minerals-model · Status: Tier-3 (fully coupled) · validated & CI-checked

This document describes the technical design of the Northern Ireland Circular

Minerals Policy Model, its data sources and provenance, and the assumptions and

limitations behind every component. It is written to be read alongside the code

in model/src/ and the data in model/data/.

For a 2–3 page non-technical, decision-maker overview of the findings, see

EXECUTIVE_SUMMARY.md.

Headline caveat. This is a *policy-scenario simulator*, not a forecasting

tool. Real, sourced anchors are used for validation and control totals; almost

all economic-structure coefficients and behavioural parameters are proxy

values, flagged as such in model/data/data_register.csv. Outputs are

illustrative of model behaviour and policy direction, not predictions. Swapping

the proxies for collected data (same interfaces) turns this into a calibrated

tool — see §8 and §9.

## 1. Purpose and scope

The model answers the Department for the Economy (DfE) minerals & circular-economy

consultation questions 2.1–2.7 by simulating the Northern Ireland minerals

system end to end:

primary extraction → processing/refining → manufacturing use → in-use stock
   → end-of-life arisings → collection → recovery/recycling → secondary material
   → (imports/exports close the balance)

It is built around circular supply security, comparing not "mining vs

recycling" but *which mix* of primary extraction, recycling, substitution,

circular design, imports and public support delivers the best balance of

resilience, GVA, employment, environmental protection and community acceptance.

Horizon: 30 years (2026–2055), discounted at the **Social Time Preference

Rate of 3.5%** (HM Treasury Green Book / NIGEAE).

## 2. Architecture

Three soft-linked engines run on an annual time step, plus a physical backbone and

indicator/spatial layers:

┌──────────────── SCENARIO / POLICY LEVER LAYER ────────────────┐
            │ grants, permitting, ESG, finance, offtake, collection,        │
            │ procurement, design standards, skills, stockpile, shocks      │
            └───────────────────────────┬───────────────────────────────────┘
                                        │ policy parameters
   ┌────────────────────────────────────▼──────────────────────────────────┐
   │ ABM (abm_module)  — agents = NAMED NI firms                             │
   │  MiningFirm · RecyclerFirm · ManufacturerFirm · GovernmentCollector     │
   │  OUT: new_domestic, recovery_boost, collection_boost, substitution      │
   └───────────────┬───────────────────────────────┬────────────────────────┘
     physical signals│                              │ behaviour ← prices/wages (CGE)
   ┌────────────────▼─────────────┐   ┌─────────────▼──────────────┐
   │ MFA (mfa_module)             │   │ CGE (cge_module) Tier-2/3   │
   │ stock-flow per mineral;      │   │ CES/CD + Armington on the   │
   │ mass balance; supply shares; │   │ NI SAM (sam_module);        │
   │ shocks (import constraint)   │   │ prices, wages, GVA          │
   └────────────────┬─────────────┘   └─────────────┬──────────────┘
   final demand (£m)│                               │ feedback (price, wage)
   ┌────────────────▼──────────────────────────────▼──────────────┐
   │ Dynamic I-O (io_module): GVA, jobs, CO₂, PM; Type I/II;        │
   │ coefficients evolve (recycling substitution, local procurement)│
   └────────────────┬──────────────────────────────────────────────┘
   ┌────────────────▼──────────────┐   ┌────────────────────────────┐
   │ Spatial (spatial_module):     │   │ Indicators (indicators):    │
   │ jobs → 11 council areas       │   │ Q2.1–2.7 + Minviro validation│
   └───────────────────────────────┘   └────────────────────────────┘

Coupling style: *soft-linking* — sequential, annual, files/objects passed

between modules — not simultaneous solving. This is the debuggable choice and lets

each module be validated independently. The annual loop lives in

coupling.CoupledModel.run() (§3.7).

### 2.1 Sectors, minerals and products

| Set | Members |
| --- | --- |
| Sectors (8) | Agriculture, Mining_Quarrying, Recycling_Secondary, Manufacturing, Energy_Utilities, Construction, Transport_Logistics, Services |
| Minerals (10) | REE_magnet, Lithium, Cobalt, Nickel, Copper, Aluminium, Antimony, Baryte, Salt, Zinc |
| Critical minerals (7) | REE_magnet, Lithium, Cobalt, Nickel, Copper, Aluminium, Antimony |
| Products | EV_battery, Permanent_magnet, Electronics_WEEE, Wind_turbine, Vehicle_ELV, Industrial_equipment |

Recycling_Secondary is split out as its own sector deliberately, so circular

activity is visible in the I-O. Critical minerals are reported separately from

bulk minerals (Salt, Baryte) because tonnage-weighting otherwise lets Salt

dominate the indicators.

## 3. Module specifications

### 3.1 Material Flow Account — mfa_module.py

Physical backbone. For each mineral *m* and year *t* it tracks a stock-flow account

and checks mass balance. Per-mineral seed parameters: `(demand0, lifetime, coll,

rec, dom0, imp_conc)`.

Flows (per mineral, per year):

End-of-life arisings: eol = inflow placed on market 'lifetime' years ago

(ring buffer _inflow_hist).

Collected: collected = eol · min(0.95, coll + collection_boost)

Recycled (secondary): recycled = collected · min(0.98, rec + recovery_boost);

supplied_secondary = min(recycled, demand)

Domestic primary: domestic_primary = demand · min(1, dom0 + new_domestic)

Imports close the balance, subject to a shock cap (§3.1.1):

remaining = max(0, demand − supplied_secondary − domestic_primary)

In-use stock updates by inflow − eol, with inflow = demand − unmet.

Mass balance (checked every year, every mineral):

domestic_primary + imports + supplied_secondary + unmet_demand = demand   (±1e-6)

Supply-security indicators (feed Q2.4 and the Vision-2035 target comparison):

domestic_share, recycled_share, import_share, supply_gap_share, and

single_country_exposure = import_share · imp_conc. Following IEA 2025 (top-three

producer concentration rose to ~86% in 2024; China refines 19 of 20 strategic

minerals at ~70% avg; export controls touch ~55% of energy-related strategic

minerals) the MFA also reports top3_exposure = import_share · top3_conc

(diversification cuts it only modestly), `refining_exposure = import_share ·

refining_conc` (diversifying *mine* imports does not reduce it — only building

domestic processing/recovery capacity does), and an export_controlled flag

(REE/antimony/cobalt). Per-mineral vectors live in policy_params.py; the coupled

run surfaces crit_max_top3, crit_refining_exposure, crit_export_control_exposure.

#### 3.1.1 Upstream supply shock (import constraint)

An optional import_constraint = {mineral: max import as a fraction of demand}

models a dominant-supplier loss (war / export ban / geopolitical fallout). If

capped imports + domestic + recycled cannot meet demand, the shortfall is recorded

as `unmet_demand` (the supply gap) rather than silently imported — this is the

threat that hits downstream firms (§5, Q2.3). Without a shock the cap is absent and

unmet = 0 (verified).

### 3.2 Agent-Based Model — abm_module.py

The ABM is firm-grounded: every agent is a named NI operator parsed from

company_register.csv (via company_data.parse_firms). 21 firms instantiate, plus

geological-potential proxy agents for tracked minerals with no named operator

(Baryte, Aluminium), plus a GovernmentCollector. Built on mesa 3.x; Tier-1 is

rule-based, Tier-2 adds adaptive expectations and peer imitation.

MiningFirm — real-option development decision each year:

eff_wacc       = WACC · (1 − 0.5·finance_support)                 # WACC = 0.1126
deposit_quality= clip((0.65·resource + 0.35·capacity)·confidence, 0, 1)
margin         = deposit_quality·price_signal + exploration_grant + 0.05·skills
                 − esg_cost − 0.12·project_risk − eff_wacc
social_licence = 0.6 − 0.5·esg_cost − 0.25·project_risk + 0.30·community_benefit
develop if  margin > dev_hurdle (0.45)  AND  social_licence > 0.4
permit delay = max(1, permit_years) + round(3·project_risk·(1 − 0.5·skills))

price_signal uses adaptive expectations when adaptive=True. Operating assets

(e.g. Irish Salt) supply immediately; proposed/exploration projects must clear the

hurdle and the social-licence floor — so the best deposit (Dalradian, high

planning-risk) can stay blocked unless a community-benefit package lifts its social

licence (the central Q2.2 finding).

RecyclerFirm — processor (e.g. Ionic → REE recovery capacity) or feedstock

collector (Re-Gen, RiverRidge, Bryson → WEEE/ELV collection):

viability = recycling_grant + recycled_content_procurement + 0.5·secondary_market
            + (price − 1) − 0.3·(energy_cost_index − 1) + 0.08·(capacity_signal − 0.5)
            + 0.04·equipment_support + 0.15·innovation_grant (+0.05 if plant exists)
gain      = min(0.06, 0.06·viability·(0.75 + capacity_signal)·(1 + 0.3·skills))
recovery_boost[m] += gain (+0.02·innovation_grant for process-yield gains)

ManufacturerFirm — downstream demand + recycled-content uptake (drives the I-O

mining→recycling coefficient shift); supply-chain firms (CDE, Terex) provide static

equipment_support that eases recycler capex:

ceiling = min(0.85, circularity + 0.15·innovation_grant + 0.05·secondary_market)
uptake  = min(ceiling, uptake + 0.5·avail·circularity + 0.05·push·circularity
              + 0.04·innovation·circ + 0.03·skills·circ + 0.05·market·circ)

GovernmentCollector — policy-driven municipal collection across critical

minerals under collection_infrastructure / product_passport.

The model emits a signals dict each step (new_domestic, recovery_boost,

collection_boost, recycling_substitution, plus firm-context aggregates) which

the coupling layer reads. CGE feedback sets dev_hurdle = 0.45·wage and feeds a

mineral price signal back to mining decisions (Tier-3 loop).

### 3.3 Dynamic Input-Output — io_module.py

Leontief production accounting with dynamic coefficients.

Leontief inverse: L = (I − A)⁻¹. A is the 8×8 proxy technical-coefficient

matrix scaled by DOMESTIC_INTENSITY = 0.62 (high import leakage of a small open

region).

Type I multipliers (direct + indirect) from L.

Type II (direct + indirect + induced) via a closed model — augment A

with a household row (income = gva_coeff·WAGE_SHARE_OF_GVA, 0.55) and column

(consumption = HH_CONSUMPTION·LOCAL_CONSUMPTION_PROPENSITY, 0.42).

Satellite accounts (per £m output): GVA, employment (jobs/£m), CO₂ (ktCO₂e),

PM (t). Impact of a final-demand vector fd: x = L·fd; multiply by satellites.

Dynamic coefficients (update_coefficients, driven by ABM each year):

recycling substitution shifts A[mine,man] → A[rec,man]; local-procurement gain

reduces mining import share (raising multipliers); a small productivity gain

shrinks all coefficients.

Calibrated so a mining demand shock yields ~10 total jobs/£m and a Type-II mining

output multiplier of ~1.69 (verified in range 1.3–2.2).

### 3.4 Social Accounting Matrix — sam_module.py

NI has no published SAM, so one is constructed and balances by construction

(Walras): structural flows set first; imports, taxes and foreign savings are

residuals.

Control total: total NI GVA ≈ £40,000m, pinned by the real mining anchor

(£108m = 0.27% of regional GVA). Sector GVA shares are proxy NI structure.

Output = gva / GVA_COEFF; intermediate flows Z = A·output; imported

intermediates are the activity-column residual.

Factor split: labour 60% / capital 40% of VA. Institutional closure: household

savings 10%, government demand share 18%, export intensities per sector.

Verified: row sums = column sums to ~1e-12, and it reproduces the £108m

mining-GVA anchor.

### 3.5 Computable General Equilibrium — cge_module.py

Compact recursive-dynamic, single-region open economy, calibrated to the NI SAM.

Value added: Cobb-Douglas in capital & labour (calibrated so VA0 = L0 + K0

replicates the benchmark exactly; labour cost share alphaL = L0/(L0+K0)).

Armington CES (Hosoe form) domestic vs imported per commodity, **σ_arm =

2.0**.

Households: Cobb-Douglas demand; fixed real government & investment.

Exports: constant-elasticity demand falling in domestic price, σ_e = 2.0.

Closure: flexible wage & capital rental clear factor markets; fixed factor

supplies; numeraire = world price = 1.

Solved with scipy.optimize.root (hybr). Benchmark replication: all excess

demands ≈ 0 at base prices (wage = 1.000). A partial-equilibrium fallback

(PEPriceLabour) is provided. (Declared SIGMA_VA = 0.8 is retained as a

parameter; the calibrated VA nest is Cobb-Douglas for exact benchmark

replication.)

Feedback to ABM (Tier-3): the relative mining price PD[mine] (clipped 0.8–1.5)

feeds next year's mineral price index; the wage index raises the mining

development hurdle (labour scarcity).

### 3.6 Spatial layer — spatial_module.py

Allocates sectoral employment to the 11 NI Local Government Districts. Base

shares follow population, tilted by sector. For the three minerals-relevant sectors

(Mining_Quarrying, Recycling_Secondary, Manufacturing) the proxy shares are

blended 50/50 with the actual named-firm geography (confidence- and

lifecycle-weighted firm headcount by district) — so mining concentrates in

Fermanagh & Omagh (Dalradian/Mannok), manufacturing in Belfast (Spirit/H&W) and

Mid & East Antrim (Wrightbus), etc. District shares sum to 1 per sector (verified).

### 3.7 Coupling loop — coupling.py

CoupledModel.run() executes the annual soft-link for t = 0 … horizon-1:

Price index = (1 + price_path)^t · cge_feedback_price; passed to ABM.

ABM step → signals (decisions).

Strategic stockpile drawdown (§5): a finite reserve tops up the import cap

each shock year until depleted.

MFA step with demand multiplier (incl. plateau), collection/recovery boosts,

new domestic supply, and the effective import constraint → physical flows.

Dynamic I-O coefficient update from ABM signals.

Convert physical flows to a final-demand vector (£m, priced); apply a

firm-grounded recycling-output floor (Ionic's installed capacity); compute

impacts (GVA, jobs, CO₂, PM).

CGE solve (if enabled) → wage, GVA; feedback to ABM.

Spatial allocation of jobs; discount by STPR; record annual row.

Demand growth optionally plateaus after a given year (demand_plateau_years,

e.g. 9 → demand grows to ~2035 then holds) so document-anchored CAGRs do not

compound unrealistically over 30 years.

### 3.8 Indicators & validation — indicators.py

Maps the time series to the seven consultation questions (§5) and validates the I-O

core against the Minviro anchors by injecting a mining final-demand shock sized to

hit Minviro's one-mine (£7.3m) and two-mine (£43m) total output, then comparing

jobs and direct mining GVA (like-with-like; Minviro's GVA is direct, not

economy-wide).

## 4. Policy levers

Every lever maps to a named real-world instrument. Default 0 (so the baseline

is undisturbed).

| Lever | Range | Acts on | Named instrument |
| --- | --- | --- | --- |
| exploration_grant | 0–~0.2 | mining margin | exploration support |
| finance_support | 0–1 | mining cost of capital | NWF + UKEF |
| permit_years | yrs | mining delay | EA priority-tracked service |
| community_benefit | 0–1 | social licence | community-benefit/ESG scheme |
| esg_cost | 0–~0.2 | margin & licence | ESG/compliance cost |
| recycling_grant | 0–1 | recycler viability | capital recovery grants (Vision 2035 DBT) |
| innovation_grant | 0–1 | recovery yield + design | CLIMATES £15m + Faraday/ReLiB £34m |
| energy_cost_index | ~0.9–1.1 | recycler viability | BICS energy support |
| secondary_market_support | 0–1 | recycler + manufacturer | UKEF offtake + price floor (cf. MP Materials) |
| collection_infrastructure | 0–1 | collection rates | WEEE/DRS |
| product_passport | 0–1 | collection + design | digital passports / EPR |
| recycled_content_procurement | 0–1 | substitution | minimum recycled-content procurement |
| design_standards | 0–1 | substitution | ecodesign / design-for-disassembly |
| local_supplier_support | 0–1 | I-O local procurement | supplier development (Invest NI) |
| skills_support | 0–1 | capacity build + mining | Skills England + DWP |
| strategic_stockpile | 0–1 | import buffer | defence stockpile (Japan/Korea practice) |
| diversification | 0–1 | single-country concentration | international partnerships (Vision 2035) — cuts effective imp_conc up to 50% |

The upstream supply shock itself (not a lever) is an import_constraint —

either a static {mineral: cap} or a callable `f(t)` for time-varying shocks

with an onset year (used by the Q2.4 Monte-Carlo). Caps are set to

1 − loss_factor·(single-country concentration).

## 5. Consultation-question experiments

Each is a standalone script writing CSVs + a findings memo to model/outputs/, and

a tab in the Streamlit app.

| Q | Script | What it does |
| --- | --- | --- |
| 2.1 Circularity innovation | q2_1_circularity_interventions.py | 7 interventions (materials recovery, secondary markets, recycling/collection, circular design, skills) individually + as a package; ranks by recycled-share lift, recycling GVA/jobs, circular-design uptake and GVA-ROI with a cost-uncertainty band |
| 2.2 Opportunities & challenges | q2_2_opportunities_challenges.py | mineral-by-mineral opportunity ranking + constraint-relaxation scenarios (permitting, finance, community/social-licence, skills, energy) → the binding constraint is social licence |
| 2.3 Business support | q2_3_business_support.py | dominant-supplier shock (per-mineral caps = 1 − concentration) + price spike; stage-targeted support (upstream/midstream/downstream/enabling); finite, depleting stockpile; severity sweep; per-mineral exposure |
| 2.4 Secure supply | q2_4_secure_supply.py | geopolitical shocks (escalating dominant-supplier export ban; caps = 1 − concentration) × five government roles; Monte-Carlo over uncertain shocks (random onset/minerals/severity) → resilience distribution + HHI-style supply-risk index vs Vision-2035 targets → the strategic-coordinator/insurer role is most robust |
| Demand & supply | q_demand_supply_strategy.py | demand scenarios from Vision 2035, EU CRMA and the UK Industrial Strategy (annex-derived CAGRs, plateau at 2035); circular-supply-chain capacity-gap analysis; ±50% demand sensitivity |

The strategic stockpile (Q2.3) is a finite reserve sized to real targets

(Japan/JOGMEC 60–180 days, Korea/KOMIR 100 days): STOCKPILE_DEPTH = 0.5

demand-years (~180 days) drawn down at up to STOCKPILE_RATE = 0.30 of demand per

shock year → ~1.5–2 years of cover, then it depletes and the gap reopens. So it is

a *bridge, not a fix*.

## 6. Data sources & provenance

Every parameter carries a status in model/data/data_register.csv:

🟢 real (sourced/audited) · 🟡 proxy (desk/scaled) · 🔴 gap (placeholder).

### 6.1 Real anchors and control totals (used for validation)

| Item | Value | Source |
| --- | --- | --- |
| NI mining & quarrying GVA (2018) | £108m (0.27% of NI GVA) | ONS Regional GVA / NISRA via Minviro |
| NI mining & quarrying workers | 1,950 | NISRA BRES |
| NI total employee jobs (Sep 2023) | 816,562 | NISRA BRES |
| Minviro one-mine scenario | 73 jobs / £7.3m output / £1.6m GVA p.a. | Minviro Final Report |
| Minviro two-mine (4b) scenario | 430 jobs / £43m output / £9m GVA p.a. | Minviro Final Report |
| STPR (discount rate) | 3.5% | Green Book / NIGEAE |
| Mining cost of equity (WACC) | 11.26% | Minviro (CAPM) |
| Vision 2035 targets (2035) | 10% domestic / 20% recycling / ≤60% single-country | Vision 2035 (GOV.UK) |
| BGS 2024 Criticality Assessment | 82 candidates → 34 critical | BGS/CMIC |
| GSNI Tellus | 55 elements geochemical baseline | GSNI |

### 6.2 Strategy-grounded demand & policy data

| Item | Value | Source |
| --- | --- | --- |
| UK growth-sector demand (Cu/Li/Co/Ni…) 2024–2035 | annex Annex-2 cumulative tonnes | Vision 2035 Technical Annex |
| Annex-derived annual demand CAGRs | Li 26.5%, Ni 10.8%, Al 9.3%, Co 9.2%, REE 8.8%, Cu 4.3% | derived from Annex 2 (cumulative → annual increment → CAGR), IEA-cross-checked |
| IEA net-zero demand | Cu/Co/REE/Ni ~2× by 2040; Li ~9× | IEA via GSNI OR25042 |
| 2023 single-country supply concentration | REE 74% China, Co 70% DRC, Li 44% Australia | BGS/Idoine et al. 2025 |
| UK metals exported for processing | ~80% | GSNI/BGS OR25042 |
| NI municipal recycling rate (2024/25) | 50.4% (household 51%) | DAERA LAC municipal waste |
| Windsor Framework CRMA applicability | Art. 13(4) | DBT Explanatory Memorandum (Apr 2025) |
| Curraghinalt (Dalradian) resource | 3.79 Moz Au M&I (+Cu/Sb/Te/Bi/Co) | Dalradian 2021 feasibility study |
| Belfast Harbour throughput (2024) | 24.1 Mt | Belfast Harbour via OR25042 |

### 6.3 Support-mechanism evidence (Q2.3)

| Item | Value | Source |
| --- | --- | --- |
| Strategic-stockpile targets | Japan 60–180 days, Korea 100 days | IEA; CSEP 2025 |
| FOAK cost-share (pilot→commercial) | 20% prototype / 50% pilot | US DOE Critical Minerals Accelerator |
| Blended finance fund | US$500m | US–Australia Critical Minerals Partnership Fund |
| Offtake + price floor | 10-yr / $110/kg NdPr | US DoD–MP Materials (CSIS 2025) |
| EU CRMA strategic-project package | 15-mo processing / 27-mo extraction permits + finance + offtake | European Commission |
| Lever public costs (NI-scale £m/yr) | CLIMATES £15m, ReLiB £34m, DBT £50m, DEFRA DRS £632m/£1.065bn, bootcamps ~£3,152/learner | Innovate UK / DEFRA / DfE |

### 6.4 Firm evidence — company_register.csv

21 named NI operators across stages, web/desk-verified, with role, district,

lifecycle, employees, investment, capacity and 0–1 scores (resource, capacity,

feedstock, demand, local-procurement, planning-risk, skills, circularity). Examples:

Ionic Technologies (Belfast, 400 tpa REE recovery), Plaswire (Lurgan,

turbine-blade composites), Dalradian/Curraghinalt, **Wrightbus, Seagate,

Encirc, Spirit, Harland & Wolff, CDE, Terex**, Re-Gen/RiverRidge/Bryson. Derived

aggregates: £1,164m named-firm capital pipeline; 400 tpa installed REE recovery.

### 6.5 Source documents

UK Critical Minerals Strategy *Vision 2035* (DBT) and its *Technical Annex*; UK

*Modern Industrial Strategy* (2025); EU *Critical Raw Materials Act* (2024); GSNI/BGS

*Critical Minerals and the Circular Economy in NI* (OR25042, 2025); *Minviro Final

Minerals Research Report* + Appendix A; BGS UK 2024 Criticality Assessment; DAERA,

NISRA, GSNI Tellus, IEA, Met4Tech/Faraday, and the cited US/EU policy analyses.

## 7. Key parameters & calibration (selected)

| Parameter | Value | Status |
| --- | --- | --- |
| DOMESTIC_INTENSITY (A scaling) | 0.62 | proxy (import leakage of a small open region) |
| GVA_COEFF (mining) | 0.35 | proxy, tuned to Minviro direct-GVA anchors |
| EMP_COEFF (mining) | 11.5 jobs/£m | proxy, anchored to 1,950 jobs / GVA |
| WAGE_SHARE_OF_GVA / LOCAL_CONSUMPTION_PROPENSITY | 0.55 / 0.42 | proxy (Type-II induced) |
| Recovery yields REE/Li/Cu | 0.85 / 0.50 / 0.90 | desk-verified (Met4Tech, ReLiB, Ionic) |
| WEEE collection baseline | 0.25 | desk-verified (UN Global E-waste Monitor) |
| Concentration imp_conc REE/Co/Li | 0.74 / 0.70 / 0.44 | real (BGS/Idoine 2025) |
| Dev hurdle / social-licence floor | 0.45 / 0.40 | proxy behavioural thresholds |
| CES elasticities σ_arm / σ_e | 2.0 / 2.0 | proxy |
| Stockpile depth / rate | 0.5 demand-yr (~180 d) / 0.30 | calibrated to JOGMEC/KOMIR |

## 8. Assumptions and limitations

Structural / economic

The I-O coefficient matrix is proxy, not a regionalised NI table. It is

scaled for high import leakage and tuned to the Minviro mining anchors. *Replace

with NISRA Supply-Use + Scottish 2017 I-O via FLQ + RAS.* This is the single

biggest calibration dependency.

The SAM structure is proxy (only mining GVA is a real anchor); the CGE is a

compact reduced-form, not a full intertemporal optimisation. CGE elasticities

are proxy.

Multipliers, GVA/output ratios and jobs/£m are calibrated to land in realistic

ranges (Type-II output ~1.4–1.8) but are not measured.

Material-flow / circular

MFA stock-flow seeds (demand, lifetimes, collection, recovery, domestic share)

are UK data scaled to NI — NI-level critical-mineral waste flows are a known

gap. Commodity prices are indicative, not market series.

Recovery yields are *process* yields; the binding constraint is *collection*

(feedstock), reflecting the reality that only ~1% of REE demand is met from

recycling.

Behavioural / ABM

Agent decision rules are deliberately simple (real-option NPV trigger, viability

scores). **Behavioural thresholds (dev hurdle, social-licence floor, risk→delay,

lever→effect coefficients) are proxy** and are what determine which marginal

projects open — the first thing to calibrate with planning/licensing and firm

survey data.

Firm scores (capacity/feedstock/planning-risk/local-procurement/circularity) are

desk estimates; Plaswire's role is verified but several firms need facility-level

confirmation.

Demand & shock

Demand CAGRs are UK-national figures scaled uniformly to NI's base; a true NI

demand series needs NI-specific sector activity (not yet public). The annex

figures are read as *cumulative* demand (reconciled against the aluminium total

and IEA); this interpretation, while well-supported, is not stated outright in

the PDF.

The supply shock is modelled as a sustained import constraint from year 0 (not a

discrete event) plus a price spike; the strategic stockpile draws down at the

release rate during shock years (a slight over-depletion if the annual gap is

smaller than the release).

Spatial / environmental

District allocation blends proxy population-tilted shares with named-firm

geography — *replace with NISRA BRES employment-by-district.*

Environmental satellites (CO₂, PM) are proxy coefficients — *replace with

NAEI/DAERA + ecoinvent/EXIOBASE.* **Q2.7 has been scoped to the consultation

question as posed — the *economic* negative impacts** (benefit leakage, closure

cliff/remediation liability, agriculture/tourism displacement, boom-bust

exposure, stranded capital; econ_impact_module.py). The *environmental*

impacts (the former pressure-index layer) are out of Q2.7 scope; the CO₂/PM

satellites still run, and a site-specific environmental assessment (Minviro

Appendix A) remains a separate exercise the model does not perform.

Public cost / ROI

Lever public costs and the GVA-ROI are NI-scale figures **benchmarked to UK

programmes by population**, used only for *relative* ranking — not budget lines.

A ±cost band is provided for sensitivity.

## 9. Validation, verification & CI

Validation against Minviro anchors (indicators.validate_against_minviro):

one-mine 7.3/73.1/£1.58m and two-mine 43.0/430.4/£9.29m — all within tolerance.

Verification harness (verify_model.py): 61 checks — *invariant*: MFA mass

balance (baseline + shock), supply-share bounds/closure, no NaN/negatives,

determinism, SAM balance (~1e-12) + mining-GVA anchor, CGE benchmark replication

(wage = 1.000), spatial share closure, stockpile reserve non-negativity/depletion,

register integrity, economic-sanity ranges, geopolitical features (diversification,

time-varying shock); plus property-based/fuzz — 30 random valid policy bundles

(random lever subsets, demand growth, static/time-varying shocks, plateau, CGE

on/off) must all preserve mass balance, share closure, no NaN/negatives and a

non-negative reserve. All 61 pass.

Continuous integration (.github/workflows/verify.yml): every push/PR

installs dependencies, runs run_mvm.py, runs verify_model.py (gates the

build) and smoke-tests all experiments. The model is deterministic (identical

config → byte-identical output; idempotent price/parameter overrides).

## 10. Reproducibility

cd model
pip install -r requirements.txt          # numpy, pandas, mesa, networkx, scipy, matplotlib, streamlit
python run_mvm.py                         # 6 scenario families → outputs/
python verify_model.py                    # 61 invariant + property-based checks
python q2_1_circularity_interventions.py  # Q2.1
python q2_2_opportunities_challenges.py   # Q2.2
python q2_3_business_support.py           # Q2.3
python q_demand_supply_strategy.py        # demand & supply
python make_plots.py                      # figures
streamlit run ../streamlit_app.py         # interactive dashboard

Environment: Python ≥ 3.11 (CI uses 3.12). Outputs are committed and deterministic.

## 11. File map

| File | Role |
| --- | --- |
| src/seed_parameters.py | sectors, minerals, proxy coefficients, real anchors/targets (read from the data register) |
| src/data_register.py | loads data_register.csv as the single source of truth |
| src/company_data.py | parses the firm register → ABM signals + stage/spatial aggregates |
| src/mfa_module.py | Material Flow Account (stock-flow, mass balance, shocks) |
| src/io_module.py | dynamic Leontief I-O (Type I/II, satellites) |
| src/abm_module.py | firm-grounded mesa agents |
| src/sam_module.py | balanced NI SAM builder |
| src/cge_module.py | recursive-dynamic CGE (+ PE fallback) |
| src/spatial_module.py | jobs → 11 council areas (firm-blended) |
| src/coupling.py | annual ABM→MFA→I-O→CGE soft-link |
| src/indicators.py | Minviro validation + Q2.1–2.7 mapping |
| data/company_register.csv | 21 named NI firms + scores |
| data/data_register.csv | every parameter → value → source → status |
| data/ree_pilot.py | REE/NdFeB magnet pilot thread |
| run_mvm.py · q2_*.py · q_demand_supply_strategy.py | entry points / experiments |
| verify_model.py | verification & validation harness |
| make_plots.py · dashboard.py · ../streamlit_app.py | figures & dashboard |

*Prepared as evidence supporting DfE minerals & circular-economy policy. All figures

are model behaviour under stated assumptions, not forecasts. See

data/data_register.csv for parameter-level provenance and status.*